| Course Name: | Elements of Electrical and Electronics Engineering | Semester: | I |
| --- | --- | --- | --- |
| Date of Performance: | 15 / 11 / 2022 | Batch No: | C2-2 |
| Faculty Name: | Jyoti Varavedkar | Roll No: | 16010122109 |
| Faculty Sign & Date: |  | Grade/Marks: | / 25 |

Experiment No: 4

Title: Thevenin’s Theorem & Norton’s Theorem.

Aim and Objective of the Experiment:

- To Verify for Thevenin Theorem for the circuit - To Verify Norton Theorem for the Circuit.

COs to be achieved:

CO1: Analyze resistive networks excited by DC sources using various network theorems. .

Circuit Diagram/ Block Diagram:

Circuit Diagram: Task 1: Circuit Diagram to measure RTH/RN: Task 2: Circuit Diagram to measure VTH: Task 3: Circuit Diagram to measure ISC:

Stepwise-Procedure:

Thevenin’s Theorem: 1. Connect the circuit as shown in the circuit diagram. 2. Set V1, V2 and measure open circuit voltage VTh across load terminals A and B. 3. Replace all voltage sources by Short circuit and measure RTh across terminals A and B as per the circuit diagram shown in the figure. 4. Draw Thevenin’s equivalent circuit and determine the value of load current from it. 5. Verify the results theoretically. Norton’s Theorem: 1. Connect the circuit as shown in the circuit diagram. 2. Set the voltages V1, V2 3. Remove the load resistance and measure the short circuit current ISC through A and B terminals. 4. Replace all the voltage sources by Short circuit and measure RTh across terminals A and B as per the circuit diagram shown in the figure. 5. Draw Norton’s equivalent circuit and determine the value of load current. 6. Verify the results theoretically

Sample Calculations:

Observation Table:

VTH In volts; RTH / RN In Ω; IN in mA. Theoretical value; 4.95; 955; 4.71. Practical value; 4.85; 950; 4.82.

Screenshot of Output (Thevenin’s and Norton’s method):

Thevenin’s equivalent circuit:; Norton’s equivalent circuit:. RTH/RN : 950 ohms; O/P for IN: 4.82 mA.

Conclusion:

By this experiment we get to know about the Thevenin Theorem and Norton Theorem for the circuit.

Signature of faculty in-charge with Date: