| Course Name: | Elements of Electrical and Electronics Engineering | Semester: | I |
| --- | --- | --- | --- |
| Date of Performance: | 22 / 11 / 2022 | Batch No: | C2-2 |
| Faculty Name: | Jyoti Varavedkar | Roll No: | 16010122109 |
| Faculty Sign & Date: |  | Grade/Marks: | / 25 |

Experiment No: 5

Title: Maximum Power Transfer Theorem

Aim and Objective of the Experiment:

- To observe maximum power transfer in D.C. circuit.

COs to be achieved:

CO1: Analyze resistive networks excited by DC sources using various network theorems. .

Circuit Diagram/ Block Diagram:

Circuit Diagram: Vs = 50 V and Rs = 500 Ω

Stepwise-Procedure:

1. Set D.C. supply voltage V= 15 V. 2. Vary RL in the range 50 Ω - 10 KΩ in steps of 100 Ω. 3. Note down IL and VL for each value of RL. Where IL and VL are current through RL and voltage across RL respectively. 4. Prepare observation table showing readings of RL Vs power P : = IL . VL 5. Plot graph of P Vs RL 6. Locate the point of maximum value of power P and note down corresponding value of RL. . Verify the results theoretically

Observation Table: Rth = 815 Ω

Sr No; RL Ω; Circuit Current (IL) mA; Power absorbed by load (PL) W PL = I2.RL. Th; Pr; Th (*10^4); Pr (*10^4). 238; 14.78; 14.68; 5.2; 4.6. 365; 12.66; 12.85; 5.8; 6.2. 557; 10.89; 11.08; 6.6; 6.6. 700; 9.87; 9.80; 6.81; 6.8. 815; 9.17; 9.25; 6.85; 6.9. 1060; 7.98; 8.20; 6.7; 6.8. 2030; 5.26; 5.28; 5.6; 5.9. 3050; 3.88; 3.82; 4.5; 4.6. 4030; 3.09; 3.11; 3.8; 3.9. 5040; 2.56; 2.56; 3.3; 3.3.

Output Snap:

Graph:

Conclusion:

By this experiment we get to know about the maximum power transfer in D.C. circuit.

Signature of faculty in-charge with Date: